- ITX-B75R1 VER:1.0

ITX-B75R1 VER:1.0 Front view

ITX-B75R1 VER:1.0 IO

ITX-B75R1 VER:1.0 Side View

| Model | ITX-B75R1 VER:1.0 |
| --- | --- |
| Processor | Support LGA1155 architecture 2th/3th Intel Celeron/Pentium/Core i3/i5/i7 CPU |
| Chipset | Intel Z77/B75 Express Chipset |
| Memory | 1*DDR3 DIMM memory slot, support 1066/1333/1600MHz, max support 8GB |
| Display | Integrated Intel® HD Graphics graphics controller |
|  | 1*VGA interface, 1*HDMI1.4 interface |
|  | Support independent display, dual display mode and extended mode |
| Network | 2* RTL8111H Gigabit network card, support network wake-up, PXE function |
| Storage | 2*SATA2.0 hard disk interface, SATA interface transmission speed can reach 3Gbps 1*MSATA solid state hard disk interface, transmission speed can reach 6Gbps |
| Audio | Integrated ALC888S high-fidelity audio controller, supports Mic, Line out |
| Expansion | 1*PCIE2.0 x16 |
|  | 1* M.2 slot, supports M.2 Key E WIFI module |
| Backplane I/O | 1*PS/2 keyboard and mouse interface |
|  | 1*VGA display interface |
|  | 1*HDMI display interface |
|  | 2*RS232 serial ports, COM1/2 supports 5V/12V power function, COM1/2 supports RS485/422 function |
|  | 2*USB 2.0 interface |
|  | 4*USB 3.0 ports |
|  | 2*RJ-45 Gigabit network interface |
|  | 1*Line out interface (green), 1*Mic interface (red) |
| Built-in I/O | 1*JVGA interface, 1*12pin pin header, same signal as external VGA |
|  | 1*PS/2 keyboard and mouse interface 1*6pin pin header |
|  | 1*Front audio interface 2*5pin pin header |
|  | 4*USB 2.0 interface 2*5pin pin header |
|  | 4*RS232 serial port 2*20pin pin header |
|  | 1*Front panel function button and indicator light interface 2*5pin pin header |
|  | 1*24pin ATX power input interface |
|  | 1*4pin ATX power input interface |
| Fan connector | 1*4pin CPU smart fan interface, 1*4pin system smart fan interface |
| GPIO | Support 4-way input and output GPIO function |
| BIOS | 32Mb Flash ROM |
| Watchdog | Support hardware reset function (256 levels, 0~255 seconds) |
| OS | Windows 7 / Linux |
| Power Type | 24Pin ATX power supply |
| Operating Temperature | -10℃～60℃ |
| Storage Temperature | -20℃～70℃ |
| Operating Humidity | 5～90% relative humidity, no condensation |
| Size | 170mm x170mm（ITX） |

- Ordering Information：

| No. | Model | Processor | Southbridge | Memory | VGA | HDMI | LAN | USB | COM | RS485/422 | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ITX-B75R1 VER:1.0 | LGA1155 | B75 | 1*DDR3 | 1 | 1 | 2*RTL8111H | 10 | 6 | COM1/COM2 | ATX |